AIM: Controlling Raspberry Pi with Telegram

Requirements:

- Hardware Requirements:

- Resistors

- LEDs

- Jumper Wires

- Breadboard

- Software Requirements:

- Raspbian Strech OS

- Telegram App

Steps:

1. Connect LEDs to GPIO pins.

Use jumper wires to connect the anode (long leg) of each LED to different GPIO pins on the Raspberry Pi.

- For example,

connect:

- LED 1 to GPIO Pin 17

- LED 2 to GPIO Pin 27

- LED 3 to GPIO Pin 22

2. Connect GND pin of raspberry Pi to GROUND of LED Module.

Connect a ground (GND) pin from the Raspberry Pi to the ground rail on the breadboard.

3.  Configure Telegram App & connect it with Rasberry Pi.

Follow the below steps to configure Telegram with Rasberry Pi.

3. Write Code in Thonny IDLE.

Code:

import random

import datetime

import time

import telepot

import RPi.GPIO as GPIO

from telepot.loop import MessageLoop

red=31

now= datetime.datetime.now()

GPIO.setmode(GPIO.BOARD)

GPIO.setwarnings(False)

GPIO.setup(red,GPIO.OUT)

GPIO.output(red,0)

def action(msg):

chat_id=msg['chat']['id']

command=msg['text']

print ('Got command:%s'%command)

if 'On' in command:

message= "Turn On"

message=message+"red"

GPIO.output(red,1)

bot.sendMessage(chat_id,"Command received");

if 'Off' in command:

message= "Turn Off"

message=message+"red"

GPIO.output(red,0)

bot.sendMessage(chat_id,"Command received");

bot= telepot.Bot('7504375765:AAEdMmGiN7H1ns9v5V7I0ifrQeskmGEskBQ')

print(bot.getMe())

MessageLoop(bot, action).run_as_thread()

print('I am listening...')

while 1:

time.sleep(10)

Steps to configure Telegram App:

Step 1: Download the TELEGRAM app

Step 2: Go to the search bar and type BotFather

Step 3: Open the BothFather’s chat & type /start

Step 4:  Then click on /newbot

Step 5: Enter username: TYIT2024_bot (the username must end with bot)

Step 6: Now we will get an access token

Step 7: Now connect the LED pins & write the code

Step 8: After writing the code, go to the terminal and type the below commands:

sudo apt-get update

sudo apt-get install python3-pip

sudo pip install telepot

python3 telegram.py (to run the program.)

Note: python telegram.py (here telegram.py is the name of our python code file)

[the last command will give you the bot details]

Step 9: Open Telegram and search for the username

Step 10: Click on the username and pass a command /start

Step 11: Write ‘On’ or ‘Off’

OUTPUT :

OUTPUT:

Command Prompt :

Development Board: